Company name/logo

Operating instructions

for fryers, woks

and deep fryers

Nummer:

Number: Please enter

Date: Please enter

Responsible: Name and telephone number

Workplace/field of activity: Enter applicable area here (e.g., department, workplace, room)

|  | - Scope |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Using fryers, woks and deep fryers |  |  |  |  |
|  | 2. Dangers to people and the environment |  |  |  |  |
|  | Burns or scalds caused by Hot grease, grease residue Hot surfaces Grease and oil burns Crushing and shearing point on mechanical lifting equipment Risk of fire from over-heated grease |  |  |  |  |
| 3. Protective measures and rules for behaviour |  |  |  |  |  |
|  | Fryers and deep fryers are only to be used by suitable people that have been trained in how to use them Check functionality of safety devices every working day after initial setup, e.g., control thermostat, extraction fans Filling Before filling the fryer, ensure that the basin is dry, that the grease/oil draining device is closed, that the heating unit is switched off and free of any residue. Only operate the heating unit if the grease/oil tray is filled below the filling line, ensuring that the heating device’s heat surface is completely covered by grease/oil. When filling with bar or block grease, this should first be liquefied in a suitable container (pot) and then carefully poured into the tray. Do not fill the tray beyond the lower or upper filling line. Operating Slowly lower/remove the basket from the hot grease/oil. Do not put food that is too wet into the hot grease/oil. Only move fryers and deep fryers once the grease/oil has cooled or if the tray has been securely closed with a tightly sealed lid. Remove any hot grease/oil that has spilled or dripped. If work has stopped or been interrupted and there is still hot grease/oil in the fryer, cover with a suitable lid and close securely. Draining and transporting grease/oil Wear heat-proof gloves with long cuffs when draining (hit) grease/oil and filters. Only drain hot grease/oil into heat-proof, sealable containers. Only transport hot grease/oil in closed containers and after sufficient cooling time. Cleaning Ensure that there is no longer any hot grease/oil in the tray. Remove any drained hot grease/oil from the workspace. Ensure that the heating device cannot be switched on during cleaning. Do not use any aggressive cleaning agents if this can be avoided. Observe the manufacturer’s manual and relevant oprating instructions. Clean draining facilities regularly in accordance with the operating conditions. |  |  |  |  |
| 4. Behaviour in case of faults |  |  |  |  |  |
|  | In case of safety-relevant faults, e.g., thermostat failure, stop using the equipment and inform your supervisor In case of grease fires: Use grease fire extinguisher – do not use water under any circumstance! |  |  |  |  |
| 5. First aid |  |  |  |  |  |
|  | Keep calm Call first aider Emergency number: 112 Report accident |  |  |  |  |
| 6. Maintenance; disposal |  |  |  |  |  |
|  | Maintenance is only to be carried out by specialists or specialist companies who have been authorised to do so. Dispose of used grease/oil in accordance with the internal disposal plan |  |  |  |  |
| Date approved: Next review date for operating instructions: |  |  | Signature: managing director/authorised person |  |  |

These sample operating instructions must be adapted for the relevant company!