|  |  | OPERATING INSTRUCTIONS In accordance with Section 14 of the German Ordinance on Hazardous Substances (GefStoffV) Scope and activities Activities involving flour | Revised 07/2023 ______________ Approved |  |
| --- | --- | --- | --- | --- |
| HAZARDOUS MATERIAL DESCRIPTION |  |  |  |  |
| ###### Respiratory sensitisers in bakeries ###### (Dust from flour and baking agents) |  |  |  |  |
| DANGERS TO PEOPLE AND THE ENVIRONMENT |  |  |  |  |
|  | - Dust from flour and baking agents can cause sensitivity and trigger allergic reactions in the form of sneezing and asthma. Occupational disease no. 4301). |  |  |  |
| PROTECTIVE MEASURES AND RULES FOR BEHAVIOUR |  |  |  |  |
|  | - Sensitising dusts should generally be avoided in breathable air. - Handling - This can be achieved by: - Using work methods that do not produce much dust during filling processes. - Handling bagged goods: Use the double-slot method wherever possible. Bags must be held and emptied as deeply into the containers being filled as they possibly can while still taking ergonomic aspects into account. - Removing flour from the silo weigher: If possible, use a filling hose that reaches the bottom or reduce drop energy by carefully guiding the house by hand. - Using baking agents in granulated, paste or liquid form. - Covering kneading machines with a tight-sealing lid. - Using work methods that do not produce much dust when preparing dough - Do not separate flour by hand, instead rub it or use a sieve. - Use low-dust flours for separating (e.g., HT flour, wheat flour, durum wheat semolina) or separating oils. Using cleaning methods that do not produce much dust - Scraper, approved vacuum, wet cleaning - It is prohibited to clean machines or floors with compressed air! - Storage - Store containers as tightly sealed as possible. |  |  |  |
| BEHAVIOUR IN CASE OF DANGER |  |  |  |  |
| - Personal protective equipment must be worn during work that creates a large amount of dust (e.g., when cleaning flour silos/extraction devices). Particle-filtering half masks must be used and must be at least level FFP2. When selecting a mask, ensure that breathing resistance is as low as possible (e.g., supported by an exhalation valve). - Phone number in case of accidents: to be filled in by company |  |  |  |  |
| FIRST AID |  |  |  |  |
|  | - In case of breathing difficulties, leave the room and prevent further exposure to dust (including from soiled work clothing), reassure the affected person and loosen/remove constricting clothing if necessary. Seek medical attention. In case of severe shortness of breath, call an ambulance/emergency doctor. - First aider: to be filled in by company |  |  |  |
| PROPER DISPOSAL |  |  |  |  |
|  | - Flour residue, empty packaging and cleaning residues must be handled so that they do not generate new dust. |  |  |  |

| Date approved: Next review date for operating instructions: | Signature: managing director/authorised person |
| --- | --- |